# 【PROITAV Multi Camera Switcher】

# Requirements Document

# P/N: CAMSW100-000

# ODM P/N:

# 修订记录 Revision History

| 版本 Version | 拟制 Prepared | 日期 Date | 备注 Notes |  |
| --- | --- | --- | --- | --- |
| V1.0 | Don | 2024-02-23 | New |  |
| V1.1 | Don | 2024-03-01 | Add NDI |  |
| V1.2 | Don | 2024-03-01 | Add description of input capabilities |  |
| V1.3 | Don | 2024-03-14 | Revised solution |  |
| V1.4 | Don | 2024-04-25 | Replied |  |
| V1.5 | Don | 2024-07-30 | Update the PRD based on the latest solution |  |
| V1.6 | Don | 2024-07-31 | Modify HDMI in to 1.4，Power supply to TBD |  |
| V1.7 | Don | 2024-08-02 | - Change the USB-C to default, USB-B as a customized option - Removed power supply specs, actual specs TBD |  |
| V1.8 | Don | 2024-08-17 | - Modify some features according to platform capacity - Add buttons |  |
| V1.9 | Don | 2024-08-26 | - Adding AI Features |  |
| V2.0 | Don | 2024-09-30 | - Remove AI Features - Add Block Diagram |  |
| V2.1 | Hornet | 2024.11.29 | - Revised according to latest template - All comments removed - All features not implemented in the 1st stage removed - Update panel layout |  |
| V2.2 | Hornet | 2025.03.19 | - Add NDI features - Update the application scenario |  |
| V2.3 | Hornet | 2025.04.08 | - HTTPS unsupported on neutral model - Power by 20V 3A PSU |  |
| V2.4 | Hornet | 2025.04.24 | - New P/N: CAMSW100-NDI-000 dedicated to NDI features |  |
| V2.5 | Hornet | 2025.07.09 | - Remove the P/N CAMSW100-NDI-000, now NDI is featured on CAMSW100-000 always. |  |
| V2.6 | Hornet | 2025.10.18 | - All multi-view layouts listed |  |
| - 项目许可 Project Approvals |  |  |  |  |
| 产品总经理 Product GM | 意见（pdf编辑区） |  |  | - 电子签名区（pdf编辑区） |

| 产品需求 Product Requirements |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 类型 Category | □矩阵 Matrix □延长器 Extender □切换器 Switcher □演示切换器 presentation switcher □分配器 Splitter □转换器 Converter □IPAV产品 IPAV product ■其他 Others Multi Camera Switcher |  |  |  |  |  |  |
| 目标市场及定位 Target Market | Personal use Small and medium conference room |  |  |  |  |  |  |
| 产品意义 Significance | Fill the gap in our product form: Convert the video signal from HDMI camera to USB UVC form, so that to used by UC system. |  |  |  |  |  |  |
| 产品详细描述 Description | The Multi Camera Switcher is an easy-to-use device that mixes up to four video sources into a single USB stream and single HDMI output. Furthermore, it supports NDI feature. It is very suitable for the scenarios such as PC recording video, video conferencing, lectures, etc. Key Features - Four 4K sources: 2 USB cameras, 2 HDMI, up to 4K@30 - Multiview supported, up to quad view - HDMI inputs support both cameras and non-cameras (BYOD screen share) - Video Bar Support (Video with Audio) - Two output video signals, the USB HOST output has mirroring HDMI output - NDI supported: - Four NDI video sources, up to 4K@30, video only - Mirroring NDI output (video only) of USB HOST and HDMI OUT2 - Multiple preset layouts: Single view, Picture in Picture, Side by Side, Left big right small, Pyramid and Quad view. - Fast seamless switching - Flexible source switching to any window - Audio In and Out, built-in audio matrix supports flexible audio routing and mixing - Layouts and source switching can be controlled by Web GUI and API via LAN or RS232. - Compatible with Windows and MacOS. Appearance Front panel. POWER; A LED with Green color On: Device is power on Off: Device is power off. STATUS; A LED with Green color On: Device is working Off: Device is not working Flash 5 times every second: upgrade or reset to factory. OLED display; OLED display to show the information 1.3 inch, 64x128 pixels White text on Black background. LAYOUT Button; 1 x push button, toggle between preset layouts. 1, 2, 3, 4 Button; 4 x push button, toggle video source of corresponding child window in the multi-view layout. AUDIO IN; 1 x 3.5mm TRS connector Analog audio in, 1Vrms. AUDIO OUT; 1 x 3.5mm TRS connector Analog audio out, 1Vrms. Rear panel. Power Input; 1 x 5.5/2.1 DC screw connector, 20V 3A or higher. LAN; 1 x RJ45 female connector 10/100/1000Mbps Ethernet NDI traffic Web Control, network API. USB IN1 USB IN2; 2 x USB 3.0 Type-A female connector USB camera input, up to 3840x2160@30, UVC and UAC specification 1A current on each. HDMI IN1 HDMI IN2; 2 x HDMI Type-A female connector Video input, up to 3840x2160@30. HDM OUT1; 1 x HDMI Type-A female connector Video output, up to 3840x2160@60 Audio output. HDMI OUT2; 1 x HDMI Type-A female connector Mirror video output of USB HOST, up to 3840x2160@30 Audio output. USB HOST; 1 x USB Type-C connector USB video output, up to 3840x2160@30, UVC and UAC specification DisplayPort Alternate mode unsupported. USB PERIPHERAL; 2 x USB 3.0 Type-A female connector USB peripherals interfaces, connected with the USB HOST port through a hub 1A current on each. RS232; 1 x 4-pin 3.5mm phoenix male connector Connected with the controller or peripheral 12V: 12V power, up to 0.5A T: TX R: RX G: GND. RESET; 1 x Push button Press shortly, show all OSD items momentary Press and hold for 5 seconds, it automatically restores to factory default settings. Device name for all parts Model name : CAMSW100 VID: 0xC021 VID: 0x1D6B Feature list Feature; Function; Description. Physical Input; Source; Four wired video sources, 2 USB 3.0 & 2 HDMI. Physical Output; To Sink; Three wired video outputs, 1 USB 3.0 & 2 HDMI. Network Input; Streaming video input; Four NDI inputs, video only. Network Output; Streaming video output; One NDI outputs, video only. Audio; Analog audio input and output Audio processing and switching. Control; RS232; Yes. API; Yes. Web GUI; Yes. USB protocol; UAC, UVC, USB HID; Yes. Compatible OS; Windows 10 or above Mac OS 10.15 or above. Accessories; 1x DC 20V 5A PSU with US cable 1x 3-pin phoenix connector for RS232 4x Mount bracket and screws. Power; DC 20V/5A. HDCP Follow sink display HDCP version information. Support HDCP 1.4, HDCP 2.2 CEC HDMI output supports sending CEC one touch play/CEC standby to the sink. CEC commands support customization on the web. Video Input HDMI IN Up to 2x 3840x2160 30Hz (2160p30) USB IN(MJPEG) Up to 3840x2160 30Hz (2160p30) for single input, up to 1920x1080 60Hz (1080p60) for dual input NDI IN(LAN port, H.264/H.265) Up to 3840x2160 30Hz (2160p30) for dual input, up to 1920x1080 60Hz (1080p60) for quad input Video output HDMI OUT1 Up to 3840x2160 60Hz (2160p60) HDMI OUT2 Up to 3840x2160 30Hz (2160p30) USB HOST (Mirror of HDMI OUT2) Up to 3840x2160 30Hz (2160p30), MJPEG NDI output (LAN port, Mirror of USB HOST and HDMI OUT2) Up to 3840x2160 30Hz (2160p30), H.264/H.265 Mutliview Up to quad view, now supports following layouts: |
| Product Feature ODM Options | Option |  |  | Notes |  |  |  |
| 开发目标Development Target | 样品数量 Samples Quantity |  |  | TBD |  |  |  |
|  | 样品要求 Samples Requirement |  |  | □基本功能机 With basic functions □全功能机 With full functions ■包装配件齐全的全功能机 With full functions and packaging parts □其他 Others |  |  |  |
|  | 送样时间 Samples Released Date |  |  | TBD |  |  |  |
|  | 产品发布时间 Launch Date |  |  | TBD |  |  |  |
|  | 其他要求 Others |  |  | N/A |  |  |  |
| 技术规格 Technical Specifications |  |  |  |  |  |  |  |
| 输入端口 Input Connectors |  | Video 2 x USB 3.0 Type-A 2 x HDMI Audio 1 x Analog stereo |  |  |  |  |  |
| 输出端口 Output Connectors |  | Video 1 x USB Type-C 2 x HDMI Audio 1 x Analog stereo |  |  |  |  |  |
| 双向端口 Bi-direction Connectors |  | USB 2 x USB 3.0 Type-A Others 1 x RS232 1 x LAN |  |  |  |  |  |
| 按键和指示灯 Buttons and LEDs |  | 2 x STATUS LED 1 x Reset button 5 x Control button |  |  |  |  |  |
| 支持的输入格式 Supported Input |  | 【Video】 HDMI (8 bit, YUV 4:4:4 / 4:2:2 or RGB, up to HDCP 1.4) 640x4808, 800x6008, 1024x7688, 1280x7688, 1280x8008, 1280x10248, 1360x7688, 1366x7688, 1440x9008, 1400x10508, 1600x12008, 1680x10508, 1920x12008, 720x4808 (480p), 720x5766 (576p), 1280x7205 (720p30), 1280x7206 (720p50), 1280x7208 (720p60), 1920x10802 (1080p24), 1920x10803 (1080p25), 1920x10805 (1080p30), 1920x10806 (1080p50), 1920x10808 (1080p60), 3840x21605 (2160p30) USB (MJPEG) 1280x7205 (720p30), 1280x7208 (720p60), 1920x10805 (1080p30), 1920x10808 (1080p60), 3840x21605 (2160p30) NDI(H.264/H.265, 8 bit, YUV 4:2:0) 1280x7205 (720p30), 1280x7208 (720p60), 1920x10805 (1080p30), 1920x10808 (1080p60), 3840x21605 (2160p30) 1 = at 23.98 Hz, 2 = at 24 Hz, 3 = at 25 Hz, 4 = at 29.97 Hz, 5 = at 30 Hz, 6 = at 50 Hz, 7 = at 59.94 Hz, 8 = 60 Hz, 9 = 75 Hz; 【Audio】 HDMI RAW PCM, 16 bit, 2-channel, 32/44.1/48KHz sps USB RAW PCM, 16 bit, 2-channel, 16/32/44.1/48/64KHz sps Analog Unbalanced, 2-channel |  |  |  |  |  |
| 支持的输出格式 Supported Output |  | 【Video】 HDMI OUT1 (8 bit, YUV 4:4:4 or RGB, up to HDCP 2.2) 720x4808 (480p60), 720x5766 (576p50), 640x4808, 800x6008, 1024x7688, 1280x10248, 1366x7688, 1440x9008, 1280x8008, 1680x10508, 1600x12008, 1920x12008, 1280x7206 (720p50), 1280x7208 (720p60), 1920x10802 (1080p24), 1920x10803 (1080p25), 1920x10805 (1080p30) , 1920x10806 (1080p50), 1920x10808 (1080p60), 3840x21603 (2160p25) , 3840x21605 (2160p30) , 3840x21606 (2160p50) , 3840x21608 (2160p60) HDMI OUT2 (8 bit, YUV 4:4:4 or RGB, up to HDCP 1.4) 1280x7205 (720p30), 1920x10805 (1080p30), 3840x21605 (2160p30) USB HOST (MJPEG) 320x2408, 640x3608, 640x4808, 1280x7205 (720p30), 1920x10805 (1080p30), 3840x21605 (2160p30) NDI(H.264/H.265, 8 bit, YUV 4:2:0) 1280x7205 (720p30), 1280x7208 (720p60), 1920x10805 (1080p30), 1920x10808 (1080p60), 3840x21605 (2160p30) 1 = at 23.98 Hz, 2 = at 24 Hz, 3 = at 25 Hz, 4 = at 29.97 Hz, 5 = at 30 Hz, 6 = at 50 Hz, 7 = at 59.94 Hz, 8 = 60 Hz, 9 = 75 Hz; 【Audio】 HDMI RAW PCM, 16 bit, 2-channel, 48KHz sps USB RAW PCM, 16 bit, 2-channel, 16KHz sps Analog Unbalanced, 2-channel |  |  |  |  |  |
| 产品USB级数设计 Product USB HUB Tier |  | 1 |  |  |  |  |  |
| 控制方式 Control |  | Button on front panel Web UI API over LAN or RS232 Reset button for reboot and factory reset |  |  |  |  |  |
| EDID 设置 EDID Setting |  | EDID can be adjusted via Web UI and API: - Fixed 4K@30Hz 2.0ch (Default). - Fixed 1080P@60Hz 2.0ch. - Fixed 720P@60Hz 2.0ch. - Customized EDID. |  |  |  |  |  |
| 功能细节 Function Details |  | - Please review the related sections of this document. |  |  |  |  |  |
| 固件升级方式 Firmware update |  | Web UI |  |  |  |  |  |
| 应用场景图 Application Scenario |  | Features at: 1) Supports up to 2 USB cameras, 2 HDMI cameras; 2) HDMI OUT1 can display the video signal of any video source; 3) HDMI OUT2, USB HOST transmit the same video signal supporting multiview (up to quad view); 4) The device can be controlled through RS232 or network connection; 5) NDI feature works, 4 NDI input and 1 NDI mirroring output. |  |  |  |  |  |
| 安装方式和位置 Installation |  | Under the table |  |  |  |  |  |
| 性能 Performance |  | Input video timing up to 2160p30Hz; Output video timing up to 2160p60Hz (HDMI OUT1); Multiview, up to quad view; NDI video stream bit rate up to 16Mbps; |  |  |  |  |  |
| 认证许可 Certification |  | 费用承担 Charge |  | （按照实际情况选择） ■ 客户承担 By Customer □ 生产商承担 By Manufacturer □ 客户和生产商分担 By Customer & Manufacturer Customer： % Manufacturer： % Notes 补充说明: |  |  |  |
|  |  | 认证类型 Certification Type |  | TBD |  |  |  |
| 认证相关资料 Certification Related Info |  | 申请方 Applicant |  | TBD |  |  |  |
|  |  | 申请方地址Address |  | TBD |  |  |  |
|  |  | 产品名称 Product |  | Multi Camera Switcher |  |  |  |
|  |  | 型号 Model No |  | TBD |  |  |  |
| 结构设计 Structural Design |  |  |  |  |  |  |  |
| 按键及接口布局图 Key and Interface Layout |  | As picture shows May vary slightly in future |  |  |  |  |  |
| 结构图（选填） Construction Drawing(Optional） |  | As picture shows May vary slightly in future |  |  |  |  |  |
| 尺寸 Dimensions |  | 250mm x 25mm x 140mm (W x H x D) |  |  |  |  |  |
| 材料 Raw Materials |  | Sheet steel |  |  |  |  |  |
| 模具 Toolings |  | 模具名称 Tooling Name | 预估费用 Estimated Cost |  | 预估周期 Lead time | 费用承担 Charge |  |
|  |  |  |  |  |  | ■客户承担 By Customer □ 生产商承担 By Manufacturer □ 客户和生产商分担 By Customer & Manufacturer Customer： % Manufacturer： % Notes 补充说明: |  |
|  |  | 以上费用合计 Total |  |  |  |  |  |
| 其他 Others |  | N/A |  |  |  |  |  |
| 表面设计 Surface Design |  |  |  |  |  |  |  |
| 丝印 Silk Screening |  | ProITAV neutral style |  |  |  |  |  |
| 表面工艺 Surfacing |  | Painting |  |  |  |  |  |
| 其他 Others |  | N/A |  |  |  |  |  |
| 包装 Packaging |  |  |  |  |  |  |  |
| 电源 Power Adapter |  | External PSU, DC 20V 3A |  |  |  |  |  |
| 贴纸 Label |  | ProITAV neutral style |  |  |  |  |  |
| 附属配件 Accessories |  | 1x 4-pin phoenix connector, for RS232 1x DC 20V 3A PSU with US AC cable 4x Mounting bracket |  |  |  |  |  |
| 说明书 User Manual |  | - ProITAV neutral style |  |  |  |  |  |
| 包装类型 Package Type |  | ProITAV neutral style |  |  |  |  |  |
| 同行产品分析 Market Analysis |  | N/A |  |  |  |  |  |