Device Discovery

  - Device Discovery Process

- Device discovery via sending multicast message

- Controller binds and listens on local port 3334

- Controller sends device search request message with IP: port = 225.1.0.0: 3333

- After receiving search request message, Device sends search response message to port 3334 of the source address (known as Controller's IP address) that requests this message.

1.2 Message Definition

1.2.1 Device Search Request Message

Table 1.1 IP Device Search Request Message Table

| Name | Type | Byte Size | Description |
| --- | --- | --- | --- |
| device_type | UInt32 | 4 | 0 any device |
| device_function | UInt32 | 4 | 0 Function_Any |

1.2.2 Device Search Response Message

Table 1.2 IP Device Search Response Message Table

| Name | Type | Byte Size | Description |
| --- | --- | --- | --- |
| device_type | UInt32 | 4 | 0 any device |
| device_function | UInt32 | 4 | 0 Function_Any |
| Reserved | Char | 32 | Reserved |
| device_name | Char | 64 | Device name (true name), ID of the device, including '\0'. = Model Name + ’-MAC’. For example:IPE970-0123456789AB. |
| Magic_number | UInt32 | 4 | Reserved |
| Protocol version | UInt32 | 4 | 2 |
| host_name | Char | 32 | Device host name, including ‘\0’. Nothing(only ‘\0’) means it follows Model Name. |
| Minor_device_name | Char | 32 | Reserved |
| service_capability | Uint32 | 4 | Reserved |
| Sub_version | UInt32 | 4 | 1 |
| vendor_id | Char | 16 | Vendor id, including ‘\0’ |
| Video_mt_st_name | Char | 25 | Reserved （IPE970 is the same as hostname） |
| Video_mt_ip_port | Char | 25 | Reserved |
| Audio_mt_st_name | Char | 25 | Reserved （IPE970 is the same as hostname） |
| Audio_mt_ip_port | Char | 25 | Reserved |

Note:

1.	device name aka true name refers to “Model name-MAC address”, for example: IPE970-0123456789AB.The device name, or true name, refers to the format “Model name-MAC address”, for Example: IPE970-0123456789AB。